# APPENDIX 7

AERODYNAMIC TESTING RESTRICTIONS (ATR)

The ATR, and the definitions and rules which will apply to aerodynamic testing, are as follows:

## General conditions

  - Restricted Aerodynamic testing is the testing by a Competitor or any Associate of a Competitor and/or by any contracted party of a Competitor or of any Associate of a Competitor or any external entity working on behalf of a Competitor or for its own purposes and subsequently providing the results of its work to a Competitor in a test environment or numerical simulation of a representation of an F1 car or sub-component in order to measure, observe or infer any forces, displacements, pressures or air flow direction resulting directly or indirectly from the incident air flow.

  - A three-dimensional representation of an F1 car or sub-component subject to Restricted Aerodynamic testing, defined either physically or digitally, will be considered for the purposes of this Article as a Restricted Aerodynamic Test Geometry (RATG) and save for where specifically permitted by this Article may not be added to, removed from, morphed or modified. In order to prevent Restricted Aerodynamic testing methodologies intended to subvert any limits on the number or nature of RATGs permitted under the ATR the following will apply:

    - The purpose of a RATG is to allow aerodynamic assessment of a single new geometry, with aerodynamic dependency maintained throughout the simulated flow field. Any attempt to derive aerodynamically independent results for subcomponents of a RATG, either in the initial simulation or test, or by subsequent modification to the simulation or test conditions, is not permitted. Any attempt to use boundary conditions or similar to infer the effect of combining RATGs, without accruing a further RATG, is also not permitted. The use of boundary conditions cannot be exploited to simulate the effect on the fluid of a geometry which is different from the RATG in use.

    - If the representation contains external bodywork surfaces on both sides of the centre plane of the car these must be symmetrical about this plane with the exception of the wheel bodywork defined by Article 3.13 of the Technical Regulations. Minimal exceptions for parts directly associated with the cooling of the power unit, or changes of car attitude allowed under sections 3d (roll and steer) and 4 will be permitted.

    - Excluding permitted degrees of freedom for RWTT and changes in RCFDs, the substitution or replacement of any part of a RATG with a non-F1 car geometry, or the placement of any physical or computational boundary condition or solver setting, that simulates or attempts to simulate a modification to this RATG will be considered as a new RATG.

    - Sections (or sub-models) of a RATG which is used for RCFDs may be created by removing geometry from the parent RATG and placing boundary conditions of velocity or pressure profiles generated entirely from the same RATG to replicate

the flow field resulting from the geometry that has been removed. This will not be considered as a new RATG provided any geometry within the section is identical to the parent RATG. Exceptionally where downstream portions of the RATG are removed these may be replaced with a single geometry approved for this purpose by the FIA.

    - Sections (or sub-models) of a RATG used for RCFDs and created by removing geometry from a parent RATG may not be subsequently geometrically modified without being considered as a new RATG. Any boundary conditions of velocity or pressure profiles added in the process of creating the section (or sub-model) and used to replicate the flow field resulting from the geometry that has been removed may not be changed to boundary condition or profiles other than those generated from the parent RATG without being considered a new RATG.

    - If the representation contains surfaces that represent components of more than one F1 car then it shall be construed as the equivalent number of RATGs. Excluding permitted degrees of freedom for RWTT and changes in RCFDs, any subsequent modification of the relative position of the representations of F1 cars will be considered as new RATGs equivalent to the number of F1 car representations.

    - A baseline RATG is defined as a reference chosen from time to time that serves for comparison purposes.

  - An Aerodynamic Testing Period (ATP) is a period of consecutive calendar weeks for the purposes of evaluation of the limits within this Article. As soon as one ATP ends a new one begins, with no gaps between them.

There will be 6 ATPs in any year. The dates of these periods will be as follows:

    - Period 1 will start on 1 January and finish at the end of week 9.

    - Periods 2, 3 and 5 will run for exactly 8 weeks each.

    - Period 4 will run for 10 weeks, comprising the Summer Factory Shutdown described in Article 24.1.

    - Period 6 will end on the 31 December.

For the above definition, weeks are assumed to start on a Monday and week 1 is the first week of four days or more in the calendar year.

In exceptional circumstances the FIA may revise these ATP at its absolute discretion in accordance with changes or events likely to affect these restrictions.

  - In the context of this Article the words bodywork and sprung suspension will have the same definition as those provided by Article 3 and Article 10 of the Technical Regulations.

Any data acquired during Restricted Aerodynamic testing may only be available to the Competitor that acquired it through use of the restricted aerodynamic testing available to it in accordance with the limits in this Article.

## Restricted Wind Tunnel Testing (RWTT)

RWTT may only be carried out in wind tunnels which have been nominated by the Competitor to the FIA. Each Competitor may nominate only one wind tunnel for use in any one twelve month period and declare it in writing to the FIA. For a new entrant, the nomination must be made no later than 7 days after the date on which it officially becomes a Competitor. No re-nominations may be made for at least 12 months. Nominations should include the facility location, unique identification of the wind tunnel and the scale of model and RATG to be used. The FIA will consider, at its absolute discretion, earlier or temporary nominations if a wind tunnel already nominated by a Competitor suffers a long term failure or for the purpose of evaluating alternative wind tunnels. If a different facility is to be used or if the existing facility is changed or upgraded, other than for routine maintenance or replacement, then a new declaration must be submitted to the FIA within one month of the change or at the time of submission of a testing period report whichever is earlier.

For the avoidance of doubt, any RWTT carried out on behalf of or for the benefit of the Competitor by an Associate, a contracted party of the Competitor or of any Associate of the Competitor or any external entity working on behalf of the Competitor or for its own purposes and subsequently providing the results of its work to a Competitor must take place in the wind tunnel nominated by the Competitor.

  - The limits for RWTT will be the number of runs of RWTT, the amount of wind tunnel occupancy time and wind tunnel wind-on time.

    - During RWTT a single run will be deemed to commence each time the wind tunnel air speed rises above 5m/s and will end the first time thereafter it falls below 5m/s.

    - During RWTT, once the wind tunnel air speed rises above 5m/s the RATG must remain fixed and unmodified until the wind tunnel air speed returns below 1m/s.

    - Between runs of RWTT detail changes to the RATG and model are permitted.

  - Wind on time is defined as the amount of time in hours summed over the ATP, where the wind tunnel air speed exceeds 15m/s for RWTT.

  - During RWTT, the first shift of occupancy will be deemed to commence the first time the wind tunnel air speed is above 5m/s on a given calendar day, and will end at a time, declared by the Competitor, when the wind tunnel air speed falls below 5m/s on the same calendar day. A second shift of occupancy will be deemed to commence the first time the wind tunnel air speed is above 5m/s following the end of the first shift of occupancy (on the same calendar day) and will end, either when the wind tunnel air speed falls below 5m/s for the last time on the same calendar day or, at the end of the calendar day in the event a run is still in progress. Only two shifts of occupancy may be carried out in any one calendar day.

  - In the event of a demonstrated wind tunnel failure or other Force Majeure the FIA will consider, at its absolute discretion, permitting additional occupancy to be used to compensate for that which is lost as a result.

  - For the avoidance of doubt any RWTT performed for the Competitor by any Associate of the Competitor and/or by any contracted party of the Competitor or of any Associate of the Competitor or any external entity working on behalf of the Competitor or for its own

purposes and subsequently providing the results of its work to the Competitor during an ATP will be subject to these same limits as if the tests were performed by the Competitor.

## RWTT Permitted technology

The following restrictions apply during RWTT:

  - Only wind tunnels that use air at atmospheric pressure as the test fluid are permitted. Other than rotations of the RATG and model or ground plane about the yaw axis, designs which attempt to create curved flow conditions relative to the RATG are not permitted. For closed section wind tunnels adaption of vertical walls and the ceiling to improve air flow uniformity is permitted. Particle image velocimetry systems where the wind tunnel air transports a flow visualisation medium are permitted.

  - No RWTT may be carried out using a scale model and RATG which is greater than 60% of full size neither may it be carried out at a wind tunnel air speed exceeding 50m/s measured relative to the scale model and RATG. Furthermore, during restricted wind tunnel testing the magnitude of the rate of change of the wind tunnel air speed measured relative to the scale model and RATG must be less than 4.5m/s². The rate of change of the wind tunnel air speed will be defined as the derivative of wind tunnel air speed and smoothed using a moving average filter, centred on each sample, of period 0.5 seconds during each wind tunnel air speed ramp up and ramp down phase. These phases are defined as the periods when the wind tunnel air speed is varying between 15m/s and 95% of the maximum wind tunnel air speed during a run.

  - Only one model and RATG may be used per run. A maximum of two models may be used and a single model change made per Competitor per 24 hour period. For the avoidance of doubt, a model in this context is defined by its underlying spine, motors and sensors.

In order to prevent testing which aims to develop for the 2026 season, from 1/12/2023 until 1/1/2025 inclusive, RWTT may only be carried out using a scale model that substantially complies with the 2023, 2024 or 2025 F1 Technical Regulations. With the exception of dyno testing aimed to develop brake system components with minimal air ducting and provided such tests do not concurrently test (or in any way provide incidental data or knowledge on) the performance or endurance of parts or systems classified as bodywork, no wind tunnel testing may be carried out using car geometry partially or wholly compliant with and/or substantially derived from drafts and/or published versions of the 2026 F1 Technical Regulations or FIA proposed 2026 bodywork geometries and concepts.

  - The only permitted degrees of freedom of the model and RATG during a run of RWTT are:

    - Wheel rotation about the wheel axis

    - Changes of ride height and roll angle relative to the ground plane and associated articulation of the elements representing the RATG suspension

    - Changes of load applied to wheels through the elements representing the RATG suspension

    - Steering of the front wheels

    - Changes of yaw angle relative to the incident air flow and/or ground plane

    - Simulation of differing exhaust flow

    - Adjustment of the flap angle of the front wing for a RATG which is substantially compliant with the 2025 Technical Regulations, or the flap angles of the adjustable regions of the Front Wing Profiles (as defined by C3.10.10 of the 2026 Technical Regulations) for a RATG which is substantially compliant with the 2026 Technical Regulations.

    - Adjustment of the incidence of the rearmost and uppermost element of the top rear wing for a RATG which is substantially compliant with the 2025 Technical Regulations, or the incidence of the RW Flap (as defined by C3.11.6 of the 2026 Technical Regulations) for a RATG which is substantially compliant with the 2026 Technical Regulations.

    - Adjustment or operation of sensors

    - Application of flow visualisation liquids

  - Changes of attitude of the model and RATG may not occur at a rate that requires changes of ride height at the front or rear axle centreline greater than the scale equivalent of 0.033 m/s on the full size F1 car and/or rotation about the yaw or roll axes at a rate greater than

1.0 deg./s.

  - Where non-rigid wind tunnel tyres are used for RWTT these may only be produced by the appointed tyre supplier. Furthermore, devices that actively modify the shape of the tyre during RWTT other than as a result of vertical and lateral loads reacted at the contact patch are not permitted. Tyre pressure control is permitted but the complete wheel must contain only a single fixed internal gas volume. Systems which apply lubricant directly or indirectly to the wind tunnel tyres in order to reduce friction at the contact patches are permitted.

## Restricted CFD (RCFD) simulations

RCFDs are Computational Fluid Dynamics (CFD) simulations by a Competitor or any Associate of a Competitor and/or by any contracted party of a Competitor or of any Associate of a Competitor or any external entity working on behalf of a Competitor or for its own purposes and subsequently providing the results of its work to a Competitor of flows that are gaseous in the case of a F1 car and are not classified as power unit simulations. Any simulation of flows contained within the power unit cooling or lubrication systems, air, air/fuel mixtures, combustion process or products of combustion from a boundary commencing at the power unit’s atmospheric air intake ducts, passing through the power unit and finishing at the exit of the exhaust tailpipe will be classified as a power unit simulation.

For the avoidance of doubt, if any CFD simulation (other than the power unit simulation defined above) reveals information to a Competitor or to an Associate of the Competitor whether directly, via a contracted party or via an external entity working on behalf of a Competitor or for its own purposes and subsequently providing the results of its work to a Competitor, about flows that are gaseous on a F1 car then it is a RCFD simulation. For example, any CFD simulations conducted at scales other than 1:1 or using non-gaseous fluids are still RCFDs as they reveal information about flows that are gaseous on the full size F1 car.

  - A RCFDs refers to the pre-processing, the solver part or parts of the simulation process, and the post processing of the results of the simulation.

    - Pre-processing refers to the meshing, decomposition and setup of the simulation.

    - Solver refers to the program or programs that compute the solution of the equations describing the flow including any extension of the simulation or simulations involving additional numerical computation (for example but not limited, to adjoint computation).

    - Post processing refers to the generation of representations of the flow solution that require numerical processing, for example but not limited to the computation of pressure coefficients, velocity, shear stress, flow streamlines or vorticity. Generation of videos or images displaying this information and any form or application of machine learning, deep learning or artificial intelligence (AI) based on simulation results is included in this definition.

During or prior to RCFDs the only permitted changes to the RATG are its attitude (ride height, roll, yaw, steer and associated tyre shape or contact patch) and front wing flap angle or rear wing rearmost and uppermost element incidence for a RATG which is substantially compliant with the 2025 Technical Regulations, or the flap angles of the adjustable regions of the Front Wing Profiles (as defined by C3.10.10 of the 2026 Technical Regulations) and incidence of the RW Flap (as defined by C3.11.6 of the 2026 Technical Regulations) for a RATG which is substantially compliant with the 2026 Technical Regulations. Furthermore, if a change in attitude causes an intersection between the Floor Bodywork and/or plank assembly and simulated ground plane, a local trim will be permitted to the Floor Bodywork and/or plank assembly only. Any trim must be the minimum required to remove the intersection, must be visible from below and must lie between XF=400 and XR=0. For avoidance of doubt, should any other changes be made to the RATG during any stage or pause in process ii. above (such as a morphing, or the addition or substitution of any boundary condition with the intent to replicate an alternative geometry) a new RATG must be counted each time a change occurs.

Changes to parts classified as LTC, TRC or OSC and contained entirely within the drum volume defined by article 3.13.2 and the scoop defined by article 3.13.3 of the Technical Regulations and outboard of YW=0 and only for the purpose of developing cooling are permitted.

Modifications to surface and volume mesh resolution and type provided they have the purpose of resolving and solving exactly the same geometry to a tolerance of 0.5mm scaled to a 1:1 car, as well as the extent of the far field domain, changing the simulation between a wind tunnel or track environment, initialisation, boundary conditions, solver settings and methodology are allowed. None of these modifications may be exploited to circumvent the requirements of the ATR by otherwise creating the effect of a change to the RATG. For the purposes of the ATR “far field” will be considered to be greater than 1m from any part of the F1 car or sub-component scaled to a 1:1 car.

A RATG may be used in RCFDs with geometry on only one side of the car centre plane, using a symmetric boundary condition on that plane, or with geometry on both sides of

the car centre plane, subject to the geometric symmetry requirements of section 1.b.ii. Changing between these two representations will not be considered a new RATG.

The addition of non-gaseous computational regions (including but not limited to coupled structural solver elements and conjugate heat transfer solid models) are not considered changes to the RATG provided that no geometric changes to the RATG itself take place during or prior to RCFDs.

The solver part or parts of all RCFDs must only be carried out using a compute resource that contains a set of homogeneous processing units and that has been nominated by the Competitor to the FIA. Each Competitor must declare to the FIA in writing the compute resources that are employed for the purpose of the solver part or parts of RCFD simulations. Floating, fixed point and integer operations from the solver part or parts of RCFD simulations must only run on and may not be offloaded from these CPU cores. From 1st January 2028 onwards, regulation will be introduced that will allow the use of GPU cores.

  - The declaration of a compute resource by the Competitor to the FIA must include:

    - The computer or cluster identification, manufacturer, model and location and the manufacturer, name and full unique model number of the Processing Units.

    - Number of processing unit cores in the compute resource.

    - Processor speed at which each Processing Unit is configured to run at 100% CPU load (CCF). In order to prevent deliberate underclocking this value may not be lower than the standard or base clock frequency given by the Manufacturer’s specification.

Any specification of compute resource declared must be available on a non-exclusive basis to all Competitors.

  - If the compute resource is changed or upgraded then a new declaration must be submitted to the FIA within one month of the change or at the time of submission of a testing period report whichever is earlier. Such changes might include, but are not limited to, a change of the hardware specification, addition or removal of processing units or change of location of any part of the compute resource.

  - The amount of compute resource used for the solve part or parts of all RCFDs shall be measured in Mega Allocation Unit hours (MAUh) and will be calculated as follows.

AUh  = (NCU * NSS * CCF) / 3600

Where:

AUh = The total number of Unit hours allocated to a CFD solver run. An Allocation Unit hour represents the use of a unit of resource allocation for one hour (and 1 x MAUh = 1,000,000 x AUh). An Allocation Unit hour is equivalent to a core hour on a physical CPU core.

CCF = Peak Processing Unit clock frequency in GigaHertz achieved during the CFD solver run. This will be the peak frequency theoretically achievable during the run based on one of the following:

- The standard or base clock frequency value from the Processing Unit

Manufacturer’s specification (if overclocking or enhanced modes are not used in the run).

- The maximum “turbo”, “HPC” or other enhanced mode frequency value.

- The maximum overclocked frequency value.

NCU = Number of Processing Unit cores used for the solver run. The effects of multi- threading, where simultaneous threads run on the same physical core will be ignored.

NSS = Number of solver wall clock seconds elapsed during the run. Message passing time during calculation must also be included.

All information required for auditing of this calculation must be present in the output from the run including the CCF value.

  - Non-RCFDs can be made by a Competitor provided that:

    - They have been requested by the FIA and use a previously-simulated RATG, and that these simulations are run as specified by the FIA and the results made available to the FIA; or

    - They use a unique RATG which has been simulated in CFD more than 30 months ago and are for the purpose of optimising CFD methodology; or

    - They use an FIA approved CAD geometry provided to all Competitors on an equitable and transparent basis and are for the purpose of optimising CFD methodology; or

    - Subject to v. and for the sole purpose of contributing toward the development of future regulations only, they use an FIA approved geometry provided for this purpose or as a basis for modification a unique RATG which has been simulated in CFD less than 30 months ago.

    - Non-RCFDs for future regulations are conditional on the full list and details of all such Non-RCFDs (including but not limited to geometries, attitudes, flow conditions) being approved by the FIA in advance of any work being carried out and full reports of the results being made available to all Competitors, via the FIA, with no team-specific Intellectual Property shared.

    - They are carried out using only the nominated compute resources described above.

    - The unique RATG or FIA approved geometry is not changed, added to, removed from, morphed or modified. Exceptions to this are permitted for the replacement of elements of the RATG or FIA approved geometry, with boundary conditions for the purposes of developing CFD sub-modelling methodology provided it does not attempt to simulate a modification to this RATG or FIA approved geometry, or where geometry changes are explicitly permitted and pre-approved by the FIA for the development of future regulations.

For the avoidance of doubt, any Non-RCFDs carried out on behalf of or for the benefit of the Competitor by an Associate, a contracted party of the Competitor or of any Associate of the Competitor or any external entity working on behalf of the Competitor or for its

own purposes and subsequently providing the results of its work to a Competitor must also meet all the requirements set out in 4.e above.

In order to prevent CFD simulations which aim to develop for the 2026 season, from 1/12/2023 until 1/1/2025 inclusive, RCFDs may only be carried out using geometries that substantially comply with the 2021-2025 F1 Technical Regulations. Except for the sole purpose of the development of brake system components and associated test rigs but excluding any development of parts or systems classified as bodywork, no RCFD simulations may be carried out using car geometry partially or wholly compliant with and/or substantially derived from drafts and/or published versions of the 2026 F1 Technical Regulations or FIA proposed 2026 bodywork geometries and concepts.

  - In the case of Non-RCFDs using a unique RATG which has been simulated in CFD more than 30 months ago or Non-RCFDs using an FIA approved geometry, geometry manipulations (e.g. in CAD clean-up or meshing software) having the sole purpose of reproducing exactly the same geometry previously solved in CFD or represented in the FIA approved CAD model or list (to a tolerance of 1.5mm scaled to a 1:1 car) are allowed. This tolerance is introduced only to allow for unintentional and incidental changes in geometry detail caused by the revisions in software and process. For the avoidance of doubt, static changes to car attitude (ride height, roll, yaw, steer and associated tyre shape or contact patch) and front wing flap angle or rear wing rearmost and uppermost element incidence (for a RATG which is substantially compliant with the 2025 Technical Regulations) are permitted, as are changes to the flap angles of the adjustable regions of the Front Wing Profiles (as defined by C3.10.10 of the 2026 Technical Regulations) and incidence of the RW Flap (as defined by C3.11.6 of the 2026 Technical Regulations) for a RATG which is substantially compliant with the 2026 Technical Regulations. Furthermore, when using an FIA approved geometry the FIA may approve additional incidence changes to bodywork (“active aerodynamics”) which will be clearly identified within the FIA approved CAD files and associated documentation.

  - Modifications to surface and volume mesh resolution and type as well as the extent of the far field domain including changing the simulation between a wind tunnel or track environment, are allowed.

  - The limits for RCFDs will be revised periodically, to take account of advances in CFD simulations.

## Exceptions to the Aerodynamic Testing Restrictions (ATR)

  - Any aerodynamic test conducted by an F1 car at any Competition or any aerodynamic test conducted by an F1 car during and at track testing as defined by Article 10 of the Sporting Regulations will not be considered as Restricted Aerodynamic testing.

  - Wind tunnel testing solely for the development of power unit heat exchangers that reject heat to air, or the running of the power unit from a boundary commencing at the power unit air intake ducts, passing through the power unit and finishing at the exit of the exhaust tailpipe will not be considered as Restricted Aerodynamic testing, provided that there is no direct or indirect measurement of aerodynamic force during the test. In this

context, pressure and flow measurements within a duct shall not be considered to be measurements of aerodynamic force.

  - Steady state and dynamic engine dynamometer work with an F1 car or subcomponent will not be considered as Restricted Aerodynamic testing provided that:

    - The bodywork used in the test has no front wing assembly as described in Article

3.9 of the Technical Regulations) or rear wing assembly (described in Article 3.10 of the Technical Regulations) present.

    - No devices designed to measure directly, or indirectly aerodynamic forces or flow field characteristics are installed in the facility used.

    - No sensor installed on the car or subcomponent which are capable of measuring displacements, pressures or air flow direction of the external airstream resulting directly or indirectly from the incident air flow may be logged. Logging files have to be available, if required, during the independent audit inspection.

    - The gas flow exiting from the exhaust system is ducted away from the testing area before impacting on any bodywork component (other than the exhaust itself).

  - Wind tunnel testing solely for the development of brake systems (Article 11 of the Technical Regulations), wheels and tyres (Article 10 of the Technical Regulations), and for development and calibration of pressure sensing instrumentation (such as pitot tubes, multi-directional probes and Kiel tubes), provided such tests do not concurrently test, or in any way provide incidental data or knowledge on, the performance or endurance of parts or systems classified as bodywork will not be considered as Restricted Aerodynamic testing. Parts classified as wheel bodywork may be fitted for Wind tunnel testing solely for the development of brake systems, wheels and tyres.

  - Wind tunnel testing that uses a RATG for the sole purpose of the conditioning of wind tunnel infrastructure or the development of wind tunnel infrastructure (including all of its sub-systems such as rolling road, model motion system, force balance, wind tunnel model spine, sensors etc.) and methodology may be performed and will not count towards the accumulation of runs, wind-on time, and occupancy subject to the testing complying with either of the following restrictions:

    - The front wing group and the rear wing group of the RATG must be removed from the wind tunnel for the duration of the testing. Alternatively, either one of or both the wing groups may be retained on the model, but each that remains must be fitted with a bluff cover that has been approved for this purpose by the FIA. The front and rear wing groups will be considered to be bodywork described by Articles

3.9 and 3.10 of the 2025 Technical Regulations respectively for a RATG which is substantially compliant with the 2025 Technical Regulations, or the bodywork described by Articles C3.10 and C3.11 of the 2026 Technical Regulations respectively for a RATG which is substantially compliant with the 2026 Technical Regulations.

    - A RATG is used which is more than 12 months old, or represents an FIA approved CAD geometry provided for this purpose and that no modification is made to this previously tested RATG or FIA approved geometry.

During audit Competitors may be requested to demonstrate compliance of any such testing through the production of supporting data.

For the avoidance of doubt, any wind tunnel testing to develop bodywork parts other than as referred to above even without aerodynamic force measurement is within the definition of Restricted Aerodynamic testing.

## Limits, Reporting, Inspection and Audit

  - The limits for RWTT and RCFDs are as set out in the tables below where:

    - P is the Competitor’s final position in the Constructors’ Championship of the previous year for the first 3 Aerodynamic Testing Periods, or the position in the current Constructors’ Championship at the end of the last day of the third Aerodynamic Testing Period, for the last 3 Aerodynamic Testing Periods.

    - C is the coefficient (expressed in percentage form) by which the various parameters need to be multiplied in order to obtain the individual RWTT and RCFD limits for each Competitor. For RWTT Runs and RATGs the result of the multiplication will be rounded up to the nearest integer.

| Wind tunnel limits for C=100%: |  |  |  | CFD limits for C=100%: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RWTT Runs | # | 320 |  | 3D new RATGs used for solve or solve part of all RCFDs | # | 2000 |
| RWTT Wind On Time | hours | 80 |  | Compute used for solve part or parts of all RCFDs | MAUh | 6 |
| RWTT Occupancy | hours | 400 |  |  |  |  |

## Coefficient C as a function of Championship position, P in 2022-2025:

| Championship Classification | P | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10+ or New Team |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value of C | % | 70 | 75 | 80 | 85 | 90 | 95 | 100 | 105 | 110 | 115 |

  - In the event that a change in ATR limits is applicable to a Competitor after the start of an ATP, the change will be applied pro-rata according to the time remaining in the ATP during which the change occurs. At its absolute discretion and following a request from the Competitor the FIA may permit over or under use of available Restricted Aerodynamic testing in the ATP during which the change occurred to be amortised or absorbed over the subsequent ATP.

  - In the event that the Championship order established at the end of the last day of the 3rd Aerodynamic Testing Period is subsequently changed following revisions to the results of a Competition or Competitions and therefore the limits applicable to certain Competitors

change, the FIA will require Competitors to adjust Restricted Aerodynamic testing in order to comply with the revised limits from the start of the next ATP. At its absolute discretion the FIA may require or permit over or under use of available Restricted Aerodynamic testing in the ATP during which the change occurred to be amortised or absorbed before the end of the year.

  - Each Competitor shall report to the FIA details of its RWTT and RCFDs for the preceding ATP within 14 days of the end of that ATP. The data must be provided in the exact format specified by the FIA, details of which may be found in the Appendix to the Technical and Sporting Regulations.

  - Digital wind tunnel image files in colour and with sufficient unobstructed field of view of the wind tunnel working section to include the entire model must be recorded, referenced to other data collected and a copy saved including a unique time stamp to at least one second accuracy for the start of each individual run.

Should the FIA wish to access the images for inspection at any time they must be of adequate quality such that it is possible to use them to verify, for example, whether the front wing group (as described in Article 3.9 of the Technical Regulations) and rear wing group (as described in Article 3.10 of the Technical Regulations) are fitted. In the case of other runs deemed to be non- RWTT in the context of this Article, for example using a RATG greater than 12 months old, or using the approved wing covers, the images must also provide a clear visual reference to assist in verifying this aspect of the test.

  - In order to permit RCFDs to continue across the end of an ATP an RATG that is used for solve or solve parts of RCFDs in an ATP may be used in any subsequent ATP subject to the requirements of Sections 1 and 4 above, without being counted again.

  - The complete surface mesh subject to the solver part or parts of each RCFD and any non- RCFD that includes a representation of an F1 car must be recorded and stored for a period of at least 24 months or until an earlier deletion is agreed by the FIA. It must be referenced to all data relating to the RCFDs or non-RCFDs including but not limited to solution monitoring data and any boundary condition of velocity and pressure profiles applied to the far field or domain boundaries and clearly and uniquely identifiable. It must be possible from these data to verify any changes made to the RATG and identify each individual flow solution generated using it. It must be possible to trace RCFDs used to generate velocity or pressure profiles applied as boundary conditions in subsequent sections or sub-models of the RATG that have not been counted as new RATGs.

  - A description correct at the start of the simulation or test of each RATG that is subject to RWTT or that is counted against the maximum permitted new RATGs for RCFDs in any ATP must be recorded with a clear description such that it is possible to easily identify the nature of the changes under evaluation. These descriptions will form part of the report required by Section 6. d) above.

  - In order to verify the Restricted Aerodynamic testing facilities employed by the Competitors and as a means of assuring common application of the restrictions set out in this Article, the FIA will arrange for independent benchmarking inspections of both wind tunnel and CFD activities to be carried out from time to time. Recommendations arising from these inspections will be incorporated into this Article.

  - Failure to comply with the limits of the ATR by a Competitor will result in a reduction of the limits that will apply to subsequent ATP or ATPs for that Competitor at the FIA’s absolute discretion but by a minimum reduction equivalent to 10 multiples of the amount by which the relevant limit or limits were exceeded without prejudice to further appropriate action. (For example, if a Competitor carries out 325 restricted wind tunnel runs against a maximum of 320 in an ATP, that Competitor shall only be permitted to make 270 restricted wind tunnel runs during the next ATP).

## Movement of personnel

No Competitor may use movement of personnel involved in the development, design or testing of aerodynamic surfaces (whether employee, consultant, contractor, secondee or any other type of permanent or temporary personnel) with another Competitor, either directly or via an external entity, for the purpose of circumventing the requirements of this Article. In order that the FIA may be satisfied that any such movement of staff is compliant with this Article, each Competitor must inform the FIA of all relevant staff movements at the end of each ATP using the template which may be found in the Appendix to the Technical and Sporting Regulations and must demonstrate that they have implemented all reasonable measures to avoid the disclosure of information, data or designs between the Competitors involved.