# APPENDIX 8

POWER UNIT TEST BENCH RESTRICTIONS

## Engine dyno test limitations

  - Definitions:

    - Engine Test Bench: An Engine Test Bench is either a Power Unit Test Bench, a Power Train Test Bench or a Full Car Dyno.

    - Power Unit Test Bench: A test bench facility cell where a fired engine with more than 1 cylinder representative of a Formula One engine may be tested. In addition to test bench components, it may include and is limited to the following power unit and car components:

      - Items listed in Technical Regulations Appendix 3 column 1 (art 5.1.2 Defin) listed as ‘INC’.

      - The clutch, flywheel, and clutch actuation system.

      - Fuel, engine oil and PU related liquids other than fuel and engine oil.

      - Heat exchangers and their associated accessories. (including but not limited to housings, tubes, pipes, hoses, supports, brackets and fasteners).

      - The PU intake upstream of compressor inlet up to and including the air filter.

      - The FIA Standard ECU.

      - The regulatory fuel flow meter (FFM).

      - Up to twelve M12 studs used to connect the ICE to the survival cell or gearbox case.

      - Additional items required for this test bench at the sole discretion of the FIA.

    - Power Train Test Bench: A test bench facility cell where a fired engine and transmission can be tested together. In addition to test bench components, it may include and is limited to the following power unit and car components in addition to those included in a Power Unit Test Bench:

      - Items listed in Technical Regulations Appendix 3 column 1 (art 5.1.2 Defin) listed as ‘EXC’.

      - A gearbox.

      - Driveshafts and any components associated with their operation (such as joints, grease and housings).

      - Heat exchangers for gearbox oil and accessories associated with their operation.

      - The complete car fuel system.

      - A survival cell or close representation of a survival cell.

      - Bodywork and air ducts or close representations of bodywork and air ducts associated with the air intake and exit of heat exchangers and compressor inlet, provided that it is for the sole purpose of representing air flow into or out of heat exchangers or the PU thermal environment.

      - Minimal suspension and steering systems required to solely and exclusively mount the survival cell and/or powertrain to the test bench facility.

      - Braking systems that are inoperable.

      - Additional items required for this test bench at the sole discretion of the FIA.

    - Full Car Dyno: A test bench facility where a fired engine can be tested, where either:

      - any car components that are not listed as included in a Power Train Test Bench are fitted or,

      - more than two power take-offs / drives are utilised, or any power take-offs / drives are utilised on the front axles.

    - Sole Purpose of Testing Power Units for Performance and Reliability: The testing of Power Units where car components or fluids supplied by each Competitor remain of the same fixed specification as the specification last used in the previous calendar year or may be changed in specification by each Competitor no more than once per calendar year with the express permission of the FIA and for the sole purpose of the evaluation of their effects on Power Unit performance and reliability. For a new supply arrangement with a Competitor a single specification of car components or fluids must be used for the entire calendar year.

Car components fitted to a Power Train Test Bench will no longer be classed as being provided for the sole purpose of testing power units for performance and reliability for the sum of all Power Train Test Bench operation for any type of test beyond the limit stated in Appendix 8 Item 1d) Limitations, irrespective of which Competitor’s car components are fitted.

Car components in this definition are all components not listed in Technical Regulations Appendix 3 column 1 (art 5.1.2 Defin) ‘INC’.

Car systems control configurations must remain unchanged other than for the sole purpose of ensuring the reliable operation of the test bench installation and with the express permission of the FIA.

    - Test Bench Occupancy Hours: Time interval between first work in the bench and last work in the bench in any 24 hours calendar day. Work can be organized in shifts but only two shifts of occupancy may be carried out in any one calendar day. The first shift of occupancy will be deemed to commence the first time the engine speed is above1000rpm on a given calendar day, or at the start of the calendar day if the engine speed is already above 1000rpm, and will end at a time, declared by the competitor, when the engine speed falls below 1000rpm on the same calendar day. A second shift of occupancy will be deemed to commence the first time the engine speed is above 1000rpm following the end of the first shift of occupancy (on the same calendar day) and will end, either when the engine speed falls below 1000rpm for the last time on the same calendar day or, at the end of the calendar day in the event a run is still in progress.

    - Test Bench Operation Hours: Time with engine speed > 7500 RPM.

    - Restricted Power Unit Testing: Restricted Power Unit Testing is any testing by a PU manufacturer, or any Related Party of that PU manufacturer, or any agent or sub-contractor of the PU manufacturer or any of its Related Parties, in a test environment of a complete or incomplete F1 PU, but always including the engine, in order to measure the torque produced by this assembly or any parameters related to the function of this assembly.

    - Annual shutdown: Period when no occupancy hours nor operations hours may be incremented.

  - Facility Limitations:

    - Allowed Power Unit Test Benches are:

Power Unit stationary in space, installed with horizontal crankshaft. No more than one power take-off / drive.

No chassis force actuators.

Test cell pressure +/-10mBar of ambient. Methods to mimic reduced ambient pressure at the engine air inlet and exhaust exits are permitted.

    - Allowed Power Train Test Benches are:

Power Unit stationary in space, installed with horizontal crankshaft. No more than two power take-offs / drives.

No chassis force actuators.

Test cell pressure +/-10mBar of ambient. Methods to mimic reduced ambient pressure at the engine air inlet and exhaust exits are permitted.

    - Allowed Full Car Dynos are:

Power Unit stationary in space, installed with horizontal crankshaft. No chassis force actuators.

Test cell pressure +/-10mBar of ambient. Methods to mimic reduced ambient pressure at the engine air inlet and exhaust exits are permitted.

  - Implementation:

    - Yearly limit divided into 10 weeks periods.

    - The time limit for a 10 weeks period may be exceeded twice by up to 20% without being in breach if the time recorded for the complete year remains within the yearly limit.

    - One single limit for all bench types (One number of test benches limit, One Occupancy hour limit, one Operation hour limit).

    - Time monitoring based on declaration based on approved software measurement (Certified software application time counter, developed by the engine test bed automation supplier).

    - Number of Engine test benches:

      - Before the 1st of December of the preceding year: Declaration from the PU supplier of the identities (Name, description, location) of Power Unit Test Benches, Power Train Test Benches and Full Car Dynos to be used in a calendar year. This includes any bench whatever their location, ownership or duration of use during the calendar year.

      - Should a Manufacturer suffer a demonstrated long-term failure of a bench or other Force Majeure the FIA will consider at its absolute discretion changes of declaration or temporary declaration.

      - No later than 2 weeks after the end of each 10 week period:

        - Designation by the PU supplier of the benches that were used for the period

        - No limitation of the number of changes of bench type (between PUTB, PTTB and FCD) per calendar year but all benches must be among the ones declared at the start of the year (unless subject to the exception for 1st July 2022 updated declaration).

      - In order to check on the hardware employed by the PU manufacturers and as a mean of assuring common application of the restrictions set out, the FIA will arrange for independent benchmarking inspections of dynos activities to be carried out from time to time.

      - Any of the Engine test benches declared above may also be used for testing any element of a power unit constructed in order to comply with the 2026 Technical Regulations once

officially ratified or, in the absence of such regulations, with the agreed technical provisions of the 2026 Championship, as determined by the FIA.

    - Test Bench occupancy and definition:

      - Sum of each individual bench occupancy hours: Total Occupancy hours: OCH

𝑛=𝑁

OCH = ∑ 𝑁𝑂𝐶𝐻𝑛

𝑛=1

With:

- N = Number of test benches

- NOCHn = Number of occupancy hours during the period for bench number n

      - Declaration 14 days after each 10 weeks period from PU manufacturers signed by top management.

      - PU manufacturers must keep records of bench tests until 1 month after the end of the calendar year

      - Photographic and component identification records must be retained for each period of occupancy that enable the FIA to confirm the Engine Test Bench definition and the purpose of testing.

    - Test Bench Operation Hours:

      - Sum of each individual bench operation hours. Total Operation hours: OPH

𝑛=𝑁

OPH = ∑ 𝑁𝑂𝑃𝐻𝑛

𝑛=1

With:

- N = Number of test benches

- NOPHn = Number of operation hours during the period for bench number n

      - Declaration 14 days after each 10 weeks period from PU manufacturers signed by top management

      - PU manufacturers must keep records of bench tests until 1 month after the end of the calendar year

  - Limitations: Base:

| Year | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- |
| Max test benches per year | 9 | 9 | 9 | 9 |
| Max Occupancy hours per year | 6000 | 4000 | 2880 | 1680 |
| Max Occupancy hours per period | 1200 | 800 | 2880 | 1680 |
| Max Overall Operation hours per year | 750 | 500 | 360 | 210 |
| Max Overall Operation hours per period | 150 | 100 | 360 | 210 |
| Max Operation hours per year on a Power Train Test Bench where car components can be classed as being provided for the sole purpose of PU performance and reliability | 150 | 100 | 100 | 100 |

This base dyno testing limitations includes the provision for any testing for a team for a calendar year (Including but not limited to gearbox, car systems, fuel and engine oil testing).

Should a PU supplier provide PU’s for use during the 2022-25 Championships to other customer Competitors who design any of their own car components as listed below or who have their own fuel or oil supplier, a quota of 30 additional operation hours will be allocated per calendar year and per additional customer Competitor. This quota to be used only during a maximum of six separate periods of up to three weeks each for the testing of Current generation PU for any combination of the components subject to agreement with the PU supplier and the FIA:

- Gearbox and/or gearbox case

- Exhaust system up to the turbine inlet and/or beyond turbine exit and WG exit

- Heat exchangers, their associated accessories and coolant header tanks

- Clutch and clutch actuation system between the PU and the gearbox

- Fuel system operating at less than 10 barG and its associated accessories

- Hydraulic system and hydraulic heat exchangers, other than servo valve(s) and actuator(s) for PU control

These additional operation hours will not be included in the sum of the operational hours stated in Appendix 8 Item 1a) ‘Sole Purpose of Testing Power Units for Performance and Reliability’ and will be specifically and exclusively for the following:

- Approving the items listed

- and / or for the sole purpose of the evaluation of the effects of the car components defined on a Power Train Test Bench on Power Unit performance and reliability.

Should an additional customer team requests to its PU supplier and the FIA to be able to run with a fuel and/or an engine oil different to the one(s) that is(are) already homologated for use with their engine; and should the PU supplier considers that this fuel and/or engine oil is(are) ready to be used in race Competitions, the PU supplier may request approval of this(ese) fuel and/or engine oil. Once this(ese) fuel and/or engine oil are approved, an extra allocation of 30 operation hours for the testing of Current generation PU may be given by the FIA specifically and exclusively for endurance validation of this(ese) fuel and/or engine oil. This extra allocation can only happen a maximum of 2 times per championship season in 2022 and 1 time per championship season from 2023 onwards.

In all cases, these additional hours must be jointly declared to FIA by the PU supplier and the customer team.

Additional occupancy hours to support the additional operation hours above will be granted at a ratio of 8:1

  - Exceptions:

ICE motoring, i.e., any activity done on a PUTB with no ignition and not showing any positive torque for the whole recorded activity, will not be counted towards the Operation or Occupancy hours of these PUTBs.

Testing activities using any element of a power unit constructed in order to comply with the 2026 Technical Regulations once officially ratified or, in the absence of such regulations, with the agreed technical provisions of the 2026 Championship, as determined by the FIA are not

counted either for occupancy or operation. Any such testing activity must not be contrived to provide benefit for any other power unit development.

## ERS dyno limitations

  - Definitions:

    - ERS Test Bench: An ERS test bench is either an ES test bench, a Power Electronics test bench and/or an MGU test bench.

    - ES Test Bench : Test bench facility cell where a complete ES representative of a Formula One ES may be tested. This excludes test bench falling into the Engine test bench definition.

    - MGU Test Bench: Test bench facility cell where an MGU (MGU-H or MGU-K) representative of a Formula One part can be tested. This excludes test bench falling into the Engine test bench definition.

    - Power Electronics Test Bench: Test bench facility cell where an ERS Control Unit, representative of a Formula One part can be tested. This excludes test bench falling into the Engine test bench definition.

    - Test Bench Occupancy Hours: Time interval between first work in the bench and last work in the bench in any 24 hours calendar day. Work can be organized in shifts but only two shifts of occupancy may be carried out in any one calendar day. The first shift of occupancy will be deemed to commence the first time the current in or out of the tested system is above 10 Amp or when MGU speed is above 1000 rpm on a given calendar day, or at the start of the calendar day if these criteria are already met, and will end at a time, declared by the competitor, when these criteria are no more met on the same calendar day. A second shift of occupancy will be deemed to commence the first time the current in or out of the tested system is above 10 Amp or when MGU speed is above 1000 rpm following the end of the first shift of occupancy (on the same calendar day) and will end, either when these criteria are no more met for the last time on the same calendar day or, at the end of the calendar day in the event a run is still in progress

    - Test Bench Operation Hours: Time with current in or out of the tested system above 10 Amps or time with MGU speed above 1000 rpm.

    - Restricted ERS Testing: Restricted ERS Testing is any testing by a PU manufacturer, or any Related Party of that PU manufacturer, or any agent or sub-contractor of the PU manufacturer or any of its Related Parties, in a test environment of a complete or incomplete F1 ERS system, but always including at least 3 out of 5 of the main ERS components (ES, CU-H, CU-K, MGU-K, MGU-H) in order to measure any parameters related to the function of this assembly. Testing falling into the scope of the Engine dyno limitation are outside this restriction. Any sign off test or end of line test are included.

    - Annual shutdown: Period when no occupancy hours nor operations hours may be incremented.

  - Facility Limitations:

    - Allowed ES Test Benches are:

Any bench where an ES can be installed and run.

This includes the benches where other PU elements can be tested as, for example, the engine. ES stationary in space.

    - Allowed MGU Test Benches are:

Any bench where an MGU can be installed and run.

This includes the benches where other PU elements can be tested as, for example, the engine. MGU stationary in space, installed with horizontal shaft.

MGU-K and MGU-H can be tested simultaneously but no more than one MGU of the same type (ie: MGU-K or MGU-H) can be tested at the same time.

It is permissible to use MGU-K and MGU-H hardware that is at least two years old within the test bench solely as a dynamometers, and these may be controlled using CU’s that are also at least two years old. These devices must be declared to the FIA

No chassis force actuators.

    - Allowed Power Electronics Test Benches are:

Any bench where an ERS Control Unit it can be installed and run.

This includes the benches where other PU elements can be tested as, for example, the engine.

CU stationary in space

  - Implementation:

    - Yearly limit divided into 10 weeks periods.

    - The time limit for a 10 weeks period may be exceeded twice by up to 20% without being in breach if the time recorded for the complete year remains within the yearly limit.

    - One single limit for all bench types (One number of test benches limit, One Occupancy hour limit, one Operation hour limit).

    - Time monitoring based on declaration based on approved software measurement (Certified software application time counter, developed by the ERS test bed automation supplier.)

    - Number of ERS Test Bench:

      - Before the 1st of December of the preceding year: Declaration from the PU supplier of the identities (Name, description, location) of ERS test benches to be used in a calendar year. This includes any bench whatever their location, ownership or duration of use during the calendar year.

      - Should a Manufacturer suffer a demonstrated long-term failure of a bench or other Force Majeure the FIA will consider at its absolute discretion changes of nomination or temporary nomination.

      - No later than 2 weeks after the end of each 10 week period:

        - Designation by the PU supplier of the benches that were used for the period

        - For Restricted Current generation ERS testing no limitation of the number of changes of bench type per calendar year but all benches must be among the ones declared at the start of the year.

      - In order to check on the hardware employed by the PU manufacturers and as a mean of assuring common application of the restrictions set out, the FIA will arrange for independent benchmarking inspections of dynos activities to be carried out from time to time.

      - Any of the ERS test benches declared above may also be used for testing any element of an ERS constructed in order to comply with the 2026 Technical Regulations once officially

ratified or, in the absence of such regulations, with the agreed technical provisions of the 2026 Championship, as determined by the FIA.

    - Test bench occupancy:

      - Sum of each individual bench occupancy hours: Total Occupancy hours: OCH

𝑛=𝑁

OCH = ∑ 𝑁𝑂𝐶𝐻𝑛

𝑛=1

With:

- N = Number of test benches

- NOCHn= Number of occupancy hours during the period for bench number n

      - Declaration 14 days after each 10 weeks period from PU manufacturers signed by top management.

      - PU manufacturers must keep records of bench tests until 1 month after the end of the calendar year

    - Test Bench Operation Hours:

      - Sum of each individual bench operation hours.

Total Operation hours: OPH

𝑛=𝑁

OPH = ∑ 𝑁𝑂𝑃𝐻𝑛

𝑛=1

With:

- N = Number of test benches

- NOPHn= Number of operation hours during the period for bench number n

      - Declaration 14 days after each 10 weeks period from PU manufacturers signed by top management

      - PU manufacturers must keep records of bench tests until 1 month after the end of the calendar year

  - Limitations:

| Year | 2022 | 2023 | 2024 | 2025 |
| --- | --- | --- | --- | --- |
| Max test benches | 4 | 4 | 4 | 4 |
| Max Occupancy hours per year | 1600 | 1200 | 880 | 480 |
| Max Occupancy hours per period | 320 | 240 | 880 | 480 |
| Max Operation hours per year | 400 | 300 | 210 | 110 |
| Max Operation hours per period | 80 | 60 | 210 | 110 |

  - Exceptions:

Testing activities using any element of an ERS constructed in order to comply with the 2026 Technical Regulations once officially ratified or, in the absence of such regulations, with the agreed technical provisions of the 2026 Championship, as determined by the FIA are not counted either for occupancy or operation. Any such testing activity must not be contrived to provide benefit for any other ERS development.